| Building/Room No.: | Date of Inspection: |
| --- | --- |
| Machine Shop Coordinator: | Date of MSC Training: |

| Compressed Gas Cylinders | Sat. | Unsat. | Comments or N/A |
| --- | --- | --- | --- |
| Inspect all compressed gas cylinders for the following: |  |  |  |
| - Storage racks properly labeled to identify cylinder contents and empty/full? |  |  |  |
| - Oxidizers and fuel gases in storage separated by at least 20 feet, by a 5-foot wall with a 30-minute fire resistance rating, or in an approved cart? |  |  |  |
| - Individual cylinders labeled as to their contents and secured to prevent tipping/falling over? |  |  |  |
| - Regulators removed and replaced with cylinder caps when not “in use”? |  |  |  |
| Overhead Cranes & Hoists | Sat. | Unsat. | Comments or N/A |
| - Rigging (i.e. slings) used for cranes and hoists in good condition and documented inspections are conducted at least annually? (no broken strands, kinking, damage, etc.) |  |  |  |
| - Crane inspected within the last year? Date of last inspection: |  |  |  |
| - Is the safe working load indicated on the hoist as well as the supporting structure (roller & beam- equal to or greater than that of the hoist)? |  |  |  |
| Electrical | Sat. | Unsat. | Comments or N/A |
| Inspect all electrical powered hand tools and cord and plug connected machinery and equipment for the following: |  |  |  |
| - Have damaged, defective, or illegal tools and equipment been removed from service? (Ex. missing ground prongs, cut/pinched cords, illegally spliced cords, etc.) |  |  |  |
| - Hand-held power tools either grounded or marked as “double insulated”? |  |  |  |
| - GFCIs used in wet or damp locations? |  |  |  |
| Inspect all electrical receptacles, disconnects, panel boxes for the following: |  |  |  |
| - Are there recognized electrical hazards that are likely to cause death or serious physical harm? (Ex. missing knockouts, missing circuit breakers, missing/broken/damaged covers, exposed live electrical components, etc.) |  |  |  |
| - Disconnecting means properly labeled (unless evident) and readily accessible? |  |  |  |
| Inspect all extension cords for the following: |  |  |  |
| - Extension cords rated for “heavy duty” (i.e no zip cords) and in good condition? |  |  |  |
| - Extension cords and power strips plugged directly into premise outlets or power strips (i.e. no daisy-chaining)? |  |  |  |
| Fire Code | Sat. | Unsat. | Comments or N/A |
| Inspect all portable fire extinguishers for the following: |  |  |  |
| - Fire extinguishers present, mounted 4’-5’ (at top) off the floor, charged, and accessible (3’ clearance)? |  |  |  |
| - Fire extinguishers inspected monthly and annually by vendor? |  |  |  |
| Inspect flammable liquids and combustibles for the following: |  |  |  |
| - Flammable liquids stored in approved flammable liquid cabinets and no combustibles (cardboard, rags, etc.) in cabinets? |  |  |  |
| Inspect for other fire code issues: |  |  |  |
| - Exits, where not obvious, marked with appropriate exit sign(s) and not blocked? |  |  |  |
| First Aid Training | Sat. | Unsat. | Comments or N/A |
| - Is there at least one person on site with current first aid training? |  |  |  |
| - Is there an adequately stocked first aid kit available? |  |  |  |
| Hazard Communication | Sat. | Unsat. | Comments or N/A |
| Inspect chemical products for the following: |  |  |  |
| - SDSs available for each hazardous product? |  |  |  |
| - Products listed above on the Chemical List? |  |  |  |
| - All primary and secondary containers properly labeled? |  |  |  |
| Hot Work | Sat. | Unsat. | Comments or N/A |
| Inspect welding/cutting areas for the following: |  |  |  |
| - Current hot work permit posted? Expiration date: |  |  |  |
| - Area maintained according to permit requirements? |  |  |  |
| Lockout/Tagout | Sat. | Unsat. | Comments or N/A |
| - Hazardous energy source(s) de-energized prior to performing work? |  |  |  |
| - Individual locks and tags applied by each authorized person? |  |  |  |
| Machinery | Sat. | Unsat. | Comments or N/A |
| (In addition to electrical issues) inspect each piece of machinery for guarding issues. NO EXPOSED BELTS, GEARS, BLADES, PULLEYS, FLYWHEELS, SHAFTS, ETC.! |  |  |  |
| - Bench Grinders Bench adjusted 1/8” from wheel Tongue adjusted ¼” from the wheel Side covers/wheel guarded |  |  |  |
| - Band Saws Blade guard lowered (or unused portion of blade guarded) No missing covers |  |  |  |
| - Lathes Properly guarded where material is secured in the chuck |  |  |  |
| - Mills Cutting head guarded |  |  |  |
| - Radial Saws Head returns to the back of the table when released Blade properly guarded and working |  |  |  |
| - Table Saws, Miter Saws, Chop Saws Blade properly guarded |  |  |  |
| - Drill Presses Bolted/secured to floor/platform |  |  |  |
| - Thermoforming Equipment All hazards identified/marked or protected |  |  |  |
| - Other (fans, garage doors, research equipment) All hazards identified/marked or protected |  |  |  |
| Personal Protective Equipment | Sat. | Unsat. | Comments or N/A |
| - PPE Hazard Assessment completed and up to date? |  |  |  |
| Powered Industrial Trucks | Sat. | Unsat. | Comments or N/A |
| - All operators currently trained and certified through EHS? |  |  |  |
| - PIT in good condition and documenting inspections prior to each use? Back up alarm working properly Lights and signals working properly Controls working properly Tires in good condition Forks in good condition |  |  |  |
| Walking, Working Surfaces | Sat. | Unsat. | Comments or N/A |
| Inspect all ladders for the following: |  |  |  |
| - Ladders rated for heavy duty (i.e. 250 lbs.) or extra-heavy duty (i.e. 300 lbs.) and inspected prior to each use for defects? |  |  |  |
| Inspect all lofts and mezzanines used for storage for the following: |  |  |  |
| - Mezzanines a part of the original structure and marked with the load rating? |  |  |  |
| - Mezzanine adequately protected from fall hazards? |  |  |  |

** Refer to EHS comprehensive audits and training for more information. **