| Date of Inspection: | Time: |
| --- | --- |
| Building and Room Location: | Competent Person: |
| Soil Classification for all Virginia Tech Soil: Class C |  |

| Site Condition | Yes | No | Action Taken | Not Applicable |
| --- | --- | --- | --- | --- |
| Has this work location been examined before the start of work operations and have all the appropriate precautions been taken? (e.g., checking for overhead objects, falling or tripping hazards, uneven ground) |  |  |  |  |
| Manufacturer’s instructions are on site and erected according to specifications? |  |  |  |  |
| Will Fall Protection be required when using this scaffold? |  |  |  |  |
| General Rules for All Scaffolds |  |  |  |  |
| Scaffold components can support at least 4x their maximum intended load. |  |  |  |  |
| Estimated intended load (includes tools, materials, etc. | ____________ Lbs. |  |  |  |
| Scaffold is fully planked with no more than 1” gap between planks. |  |  |  |  |
| Platform is at least 18 inches wide (12” on pump jacks) |  |  |  |  |
| Guardrails are used if work height is >6 feet. |  |  |  |  |
| Scaffold is 14” or less from face of work (If guardrails are not feasible). (18” for plasterers) |  |  |  |  |
| Planks must extend at least 6” past the bearer and no more than 12 to 18” depending on the span of the board. Cleating or other means of securing is always recommended. |  |  |  |  |
| Work platform is free of clutter, mud, oil, or any tripping hazard. |  |  |  |  |
| Scaffolding, materials, and personnel are more than 10’ from any power line. |  |  |  |  |
| Are wheel castors locked if applicable? |  |  |  |  |
| If any piece of scaffold is defective, it has been removed from serve and tagged. |  |  |  |  |
| Rules for Supported Scaffolds |  |  |  |  |
| Height to base with ratio is less than 4:1 (no ties, guys, or braces required) |  |  |  |  |
| If ratio is more than 4:1, scaffolds are restrained from tipping by tying or bracing. |  |  |  |  |
| All scaffold frames and uprights use base plates, (mudsills if on dirt). |  |  |  |  |
| Footings are level, sound, and rigid. No settling has occurred. |  |  |  |  |
| Unstable objects such as blocks, bricks, buckets, etc. are not used as work platforms or to support scaffolds. |  |  |  |  |
| General Rules for Access |  |  |  |  |
| End frames (where permitted by the manufacturer) used for access have rungs lined up vertically for the entire height of the scaffold. |  |  |  |  |
| There is no more than 2’ step up or down or a 14” step across to get on or off a platform. |  |  |  |  |
| First rung of the ladder is not more than 24” above the ground. |  |  |  |  |
| Proper access has been provided (i.e. ladder or frame designed by manufacturer to be climbed). |  |  |  |  |
| Cross braces are not used for climbing up or down from the scaffold. |  |  |  |  |